# Test Fixture GUI - Speaker Notes

Casual talking points, one block per slide. Read them like you'd say them. Times are rough.

## 1 - Title

Quick intro: this is the test fixture GUI I've been building. I'll walk through what it does, demo a couple of the test windows live, then get into how it's actually built and where things stand. Should be about ten minutes.

## 2 - What it is

Big picture: it's one app an operator runs at the bench. It drives a precision Zaber actuator into a sensor while reading a FUTEK load cell, and every test, every safety check, and the results all live in one window. The key idea is that it's closed-loop: the GUI moves the actuator based on the live force reading, not a fixed script.

## 3 - Project goals

Before the demos, the goals the GUI was built around. The line up top is the mission: a platform to configure, run, monitor, and analyze sensor tests without giving up ease of use, safety, data integrity, or efficiency. The eight goals underneath are the specifics: intuitive to operate, safe with the hardware, every test traceable to a sensor, efficient, clear to monitor, covers all four test modes (EM, Shear, Manual, Cyclical), and built to extend and maintain. The appendix at the end has the detailed bullets under each goal if anyone wants to drill in.

## 4 - Where the project is

Where things stand: five test workflows all run end to end, there are six safety systems built in, and it's documented with over a hundred user stories across eleven features. It's an MVP, packaged as a double-click app on Mac and Windows. The piece that's still in progress is the formal test pass.

## 5 - Divider: What I built

Let me start with what the GUI actually does for the operator.

## 6 - Five test workflows

There are five test types. EM is the main one: press the sensor and read its response against pressure. Calibration / Fuji is the Fuji-film setup. Manual is free actuator control for exploring. Shear is load-cell-only, the operator pushes by hand. And Fatigue is cyclical loading, thousands of cycles.

## 7 - Demo: Fatigue window

This is a good one to show live. You pick a waveform - sine, square, triangle, sawtooth, or a blood-pressure pulse - set the force bounds and frequency, and it draws a live preview of exactly what the actuator will do. It even estimates the test length, here about eight hours for 28,800 cycles. And there's spike protection: if the force ever jumps past the configured swing, it stops.

If presenting live: open the Fatigue window, change the frequency, and let them watch the preview redraw.

## 8 - Concept: fatigue waveform builder

This one is a concept, a mockup of where the fatigue window could go next, so flag it as not-yet-built. The idea: a fatigue test is just pressing between a low and a high force, so Lower, Upper, frequency, and cycles always mean the same thing. The waveform only changes the shape of the press in between. Most operators just pick a preset, sine, square, triangle, sawtooth, or a blood-pressure pulse, and never touch math. Blood pressure adds a couple of sliders, heart rate, dicrotic notch depth, and upstroke sharpness, so you can dial a realistic arterial pulse. The clever part is the warning banner: it works out the steepest part of the shape in newtons per second and compares it to what the actuator can physically do, so it tells you up front when a curve is too sharp to follow on the rig. That is the same reason a square wave comes out looking like a sine on real hardware. And for power users there is a custom equation box with a safe parser, type any shape in terms of phase and it auto-scales into your bounds.

Two honest caveats to say out loud: it is a mockup, not wired into the app yet, and the actuator speed limit in it is a placeholder until it is measured on the rig. If presenting live: open the standalone page, switch a smooth shape to square, and watch the banner flip to the warning.

## 9 - Demo: Manual window

The manual window is the most flexible. You can move by a set distance, or, the cool part, drive until the load cell reads a target force, both pressing down and releasing back up. Force records continuously the whole time the window is open, and you can tweak the graph view mid-move while the motion controls stay locked for safety.

## 10 - Demo: EM test and run management

EM is the workhorse. Under the hood every run is a strict state machine, and you can see each state timestamped in the status log. It auto-runs multiple presses back to back, pausing between so you can reset the sensor. And redo never destroys data: redoing a run creates a new run number and supersedes the old one, so the original is always kept.

## 11 - Calibration and Fuji film

Calibration and the Fuji film test. The newer piece is the extrusion-distance field: the operator sets how far the actuator extrudes, capped at 12 mm, and it remembers the last value used. There's a fine jog for nudging position, and I fixed a nasty bug where the window thought the stage was still moving after a jog had finished.

## 12 - Safety, in every test

Safety is the part I'm most careful about, since this drives real motors into a load cell. Six systems: a hard force ceiling, spike detection for metal-on-metal contact, travel limits, disconnect handling, snap-to on every input field, and the single-owner load-cell reader. These are built into every test, not bolted on per feature.

## 13 - Limits, bounds, and safety stops

This is the reference slide, the actual numbers behind the safety story. The left side is everything the operator can set, and every one of those snaps into range the moment you commit it, so you can type freely while editing but you can't leave a bad value in. The right side is the fixed stuff nobody can touch: the stage travels seventeen to fifty point eight millimeters, the actuator physically tops out around twenty-five newtons, and the hard safety ceiling is thirty-three newtons, where a press stops instantly and homes itself. The sudden-jump cutoff scales with speed, so a slow press trips at a gentle force and a fast one tolerates more, that's what catches metal-on-metal contact. And two things that look like limits but aren't: the thirty-two newton EM target and the twenty newton Fuji target are just where a good press stops, not safety caps.

Reconciliation note, only if someone digs in: the load cell is rated about 22 N, but the force input fields currently cap at 32 N (with the 33 N safety ceiling). Whether to tighten the input cap to the load-cell rating is an open item between the limits doc and the code.

## 14 - Divider: Under the hood

Okay, let me get into how it's actually built, in case the architecture is interesting.

## 15 - Architecture: how it's built

It's three layers. The front end is plain HTML, CSS, and JavaScript, one file per test tab, no framework. That talks to a small local Python server over JSON, and the browser just polls a status endpoint many times a second to draw the live graphs. The server owns a test engine that runs each test on a background thread, and the engine talks to the hardware drivers, Zaber over serial and the FUTEK load cell over .NET. The whole thing packages into one double-click app with PyInstaller.

## 16 - The hardest bug: the load cell

If I had to pick the hardest bug, it was this. The load cell kept dropping to simulated data partway through a session, and once it dropped, every test after it read fake values too. It turned out the manual window's live graph and the running test were both reading the device at the same time on different threads, and the driver cannot be read twice at once, so it corrupted. The fix was to make exactly one background thread own the device and have everything else read its cached value. Two simultaneous reads are now impossible.

## 17 - Is the live graph behind the real force?

This is the question I would expect about the fast tests. Honest answer first: yes, during a quick fatigue cycle the on-screen trace can sit a few tens of milliseconds behind the true load, because every sample has to travel from the load cell, through the backend, across the pywebview bridge, and wait for a redraw. But that lag does not put the test at risk, and here is the why. The safety cutoffs, the spike check and the force ceiling, run on the raw sample stream in the backend, before a single point is ever drawn, so display lag can never delay a safety stop. The actuator is driven from the same backend waveform, not from the screen, so the graph is just a read-only mirror. That makes the lag a UX number: it changes how live the graph feels, never whether the test is safe or the saved data is right.

If pressed on numbers, go to the next slide and show the simulator.

## 18 - Live graph latency simulator

To keep myself honest about that number I built a little standalone page, it opens in any browser with no backend running, that models the whole signal path. You set the sample rate, how samples are pushed, the bridge overhead, and the redraw cadence, and it estimates the display lag and shows the true signal against the lagging on-screen one. A typical setup lands around 60 ms, somewhere in the 14 to 107 ms range, which is streaming, not hard real time. The key caveat on the slide: this is a model, not a measurement. To get the real figure for a given machine you stamp the sample with perf_counter() in Python when it is acquired, record performance.now() in the draw callback, take the difference, and report the median and the 95th percentile over a 30 to 60 second run. The 95th percentile is the honest worst case to quote, not the average.

If presenting live: open the simulator, drag the redraw cadence or batch interval slider, and watch the lag number and the two traces respond.

## 19 - Live force graph: values

The next four are reference slides, the documentation behind the numbers, so I will go quick unless you want detail. This first one is just the live force graph's properties: it plots force in newtons against time, samples at 100 Hz, and the display lag is roughly 60 ms typical. The values are estimates for now; the plan is to swap in measured numbers once I profile it on the rig.

## 20 - Live force graph: lag breakdown

This breaks that 60 ms into where it actually goes. The biggest chunks are the transport wait and the frame wait, the two waiting-for-the-next-turn stages, while the backend and bridge are tiny. The line at the bottom is just the sum: each waiting stage contributes about half its interval on average.

## 21 - Fatigue simulator: force calculation

This documents how the fatigue simulator turns a shape into a force. You take the shape, normalize it to a 0-to-1 range, scale it into your lower and upper bounds, and top it off at the 33 N ceiling. The three lines of code at the bottom are the whole force calculation.

## 22 - Fatigue simulator: round-off check

And this is the round-off, or slew, check. It measures the steepest the force ever needs to change, in newtons per second, and compares it to what the actuator can physically do. The examples make it concrete: a slow sine is fine, but a blood-pressure pulse and a square wave are too steep, so they round off on the rig. The limit is block stiffness times max actuator speed, and the 60 N/s is a placeholder until the block stiffness is measured.

## 23 - Day by day

Quick timeline. The first couple of weeks were clinical data work; the GUI really starts at Day 19 with documenting the engine. Then it's motion and serial reliability, safety and UI state, run management, and then the big hardware-behavior and load-cell days. The most recent stretch was the snap-to input limits, the limits-and-bounds reference, reconciling the user stories, and then writing the formal test plan, forty-five EM test cases, which just wrapped. A couple of days in there were other-track work, the data script and the website.

## 24 - It's all tracked in one workbook

Quick credibility slide. None of this lives only in my head: there's one documentation workbook, fourteen sheets, that tracks everything. A verification checklist of eighty-plus pass/fail checks across every area, which is what drives the formal test pass. A GUI updates log with sixty-plus entries, every visual, backend, hardware, and documentation change, with the files it touched and whether it's tested. A user-story tracker with over a hundred stories and their acceptance criteria. Plus the limits reference and the folder workflow. So anyone can pick this up and see exactly what was done and what still needs verifying.

## 25 - Where it stands and what's next

Where it stands: every test type runs end to end, the safety behaviors are verified in simulation, and it's verified on the rig where I had hardware. What's next is executing the formal test plan, confirming the safety-critical cases on the real rig with deliberate fault injection (unplugging cables, pressing into metal), and reconciling the user-stories doc to the current GUI.

## 26 - Resources

And it's all documented: the user stories, the formal test plan, and the code is version-controlled. Happy to dig into any layer. Questions?

## Appendix - Project goals (talking points)

If someone asks what the project was actually aiming for, these are the eight goals and the specifics under each.

The EM Test Fixture GUI should provide a unified platform for configuring, executing, monitoring, and analyzing sensor characterization tests while maintaining ease of use, operator safety, data integrity, and workflow efficiency.

### Goal 1: Intuitive User Experience

The GUI should be easy to learn and operate with minimal training.

- Testing workflows should follow a logical progression

- Related settings should be grouped together

- Important actions should be clearly labeled

- Informational tooltips should explain technical settings

- Users should be able to understand the current state of the test at all times

### Goal 2: Safe Hardware Operation

The GUI should reduce the risk of accidental hardware misuse or unsafe actuator movement.

- User inputs should be validated before testing begins

- Hardware initialization should occur before motion commands

- Unsafe operations should be blocked

- Home / Reset functions should be easily accessible

- Clear status messages should indicate hardware state and errors

### Goal 3: Data Integrity and Traceability

All test data should be organized, reproducible, and traceable to a specific sensor and test configuration.

- Each test should be associated with a Sensor ID

- Test outputs should be automatically saved

- Folder structures should be generated consistently

- Existing test data should be protected from accidental overwrites

- Analysis outputs should be linked to the originating test

### Goal 4: Efficient Testing Workflows

The GUI should minimize repetitive user actions and streamline sensor characterization.

- Common settings should be remembered when appropriate

- Folder creation should be automated

- Test setup should require minimal manual configuration

- Existing test workflows should support reruns and versioning

- Analysis should be accessible directly from testing windows

### Goal 5: Clear Test Monitoring

Users should be able to understand test progress and system status in real time.

- Active test status should be visible

- Graphs should update live when applicable

- Run progress should be clearly displayed

- Error conditions should be communicated immediately

- Analysis results should be presented in an organized manner

### Goal 6: Flexible Sensor Characterization

The platform should support multiple testing modes (EM, Shear, Manual, and Cyclical) while maintaining a consistent user experience.

- Supported testing modes: EM Testing, Shear Testing, Manual Testing, Cyclical Testing

- Each testing mode should follow consistent navigation, status reporting, and data management principles

### Goal 7: Extensibility

The software should be designed so future testing methods and analysis tools can be added without major redesign.

- Examples: additional waveform types

- Blood-pressure waveform simulation

- Automated pass / fail criteria

- Sensor database integration

- Additional analysis metrics

- Future hardware platforms

### Goal 8: Maintainability

The software should be organized so future developers can understand, modify, and extend the system.

- Workflows should be clearly documented

- User stories should map directly to GUI functionality

- Validation logic should be consistent throughout the application

- Analysis outputs should be standardized across test types